|  | Simulation of Ammo Base Date: Monday, May 10, 2021 Designer: Cameron Rosenberger Study name: Static 2 Analysis type: Static. Table of Contents Description 1 Assumptions 2 Model Information 3 Study Properties 4 Units 5 Material Properties 5 Loads and Fixtures 6 Connector Definitions 7 Contact Information 7 Mesh information 8 Sensor Details 10 Resultant Forces 11 Beams 11 Study Results 12 Conclusion 17. |
| --- | --- |
| # Description No Data |  |

# Assumptions Original Model; Model Analyzed.

# Model Information Model name: Ammo Base Current Configuration: Default. Solid Bodies. Document Name and Reference; Treated As; Volumetric Properties; Document Path/Date Modified. Fillet6; Solid Body; Mass:1.26292 kg Volume:0.00046775 m^3 Density:2,700 kg/m^3 Weight:12.3767 N; \\apporto.com\dfs\SMU\Users\47865731_smu\Documents\Ammo Base.SLDPRT May 9 17:42:07 2021.

# Study Properties Study name; Static 2. Analysis type; Static. Mesh type; Solid Mesh. Thermal Effect:; On. Thermal option; Include temperature loads. Zero strain temperature; 298 Kelvin. Include fluid pressure effects from SOLIDWORKS Flow Simulation; Off. Solver type; FFEPlus. Inplane Effect:; Off. Soft Spring:; Off. Inertial Relief:; Off. Incompatible bonding options; Automatic. Large displacement; Off. Compute free body forces; On. Friction; Off. Use Adaptive Method:; h-Adaptive. Result folder; SOLIDWORKS document (\\apporto.com\dfs\SMU\Users\47865731_smu\Documents). ## h-Adaptive Options Target accuracy; 98 %. Accuracy bias; 50. Maximum no. of loops; 3. Mesh coarsening; Off.

# Units Unit system:; SI (MKS). Length/Displacement; mm. Temperature; Kelvin. Angular velocity; Rad/sec. Pressure/Stress; N/m^2.

# Material Properties Model Reference; Properties; Components. Name:; 1060 Alloy. Model type:; Linear Elastic Isotropic. Default failure criterion:; Unknown. Yield strength:; 2.75742e+07 N/m^2. Tensile strength:; 6.89356e+07 N/m^2. Elastic modulus:; 6.9e+10 N/m^2. Poisson's ratio:; 0.33. Mass density:; 2,700 kg/m^3. Shear modulus:; 2.7e+10 N/m^2. Thermal expansion coefficient:; 2.4e-05 /Kelvin.; SolidBody 3(Fillet6)(Ammo Base). Curve Data:N/A.

# Loads and Fixtures Fixture name; Fixture Image; Fixture Details. Fixed-3; Entities:; 1 edge(s), 3 face(s). Type:; Fixed Geometry.. Resultant Forces Components; X; Y; Z; Resultant. Reaction force(N); -0.0279882; 222.37; -0.0605838; 222.37. Reaction Moment(N.m); 0; 0; 0; 0.. Load name; Load Image; Load Details. Force-2; Entities:; 1 face(s). Type:; Apply normal force. Value:; 50 lbf.. Force-3; Entities:; 4 face(s). Type:; Apply normal force. Value:; 25 lbf..

# Connector Definitions No Data

# Contact Information No Data

# Mesh information Mesh type; Solid Mesh. Mesher Used:; Standard mesh. Automatic Transition:; Off. Include Mesh Auto Loops:; Off. Jacobian points for High quality mesh; 16 Points. Element Size; 0.42068 in. Tolerance; 0.021034 in. Mesh Quality; High. ## Mesh information - Details Total Nodes; 73818. Total Elements; 41384. Maximum Aspect Ratio; 19.945. % of elements with Aspect Ratio < 3; 17.8. Percentage of elements with Aspect Ratio > 10; 2.7. Percentage of distorted elements; 0. Time to complete mesh(hh;mm;ss):; 00:00:02. Computer name:; NV-SMU1-GPU1.

# Sensor Details Sensor name; Location; Sensor Details. Displacement1; Value : 6.570e-03 mm Entities : Result :Displacement Component :URES: Resultant Displacement Criterion :Model Max Step Criterion : Across all Steps Step No.:1 Alert Value: NA. Stress1; Value : 1.807e+06 N/m^2 Entities : Result :Stress Component :VON: von Mises Stress Criterion :Model Max Step Criterion : Across all Steps Step No.:1 Alert Value: NA.

# Resultant Forces ## Reaction forces Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N; -0.0279882; 222.37; -0.0605838; 222.37. ## Reaction Moments Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N.m; 0; 0; 0; 0.

## Free body forces Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N; 2.29546; -0.031976; 1.8146; 2.92625. ## Free body moments Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N.m; 0; 0; 0; 1e-33.

# Beams No Data

# Study Results Name; Type; Min; Max. Stress1; VON: von Mises Stress; 2.203e+03N/m^2 Node: 38403; 8.871e+06N/m^2 Node: 20202. Ammo Base-Static 2-Stress-Stress1. Name; Type; Min; Max. Displacement1; URES: Resultant Displacement; 0.000e+00mm Node: 9; 1.657e-01mm Node: 562. Ammo Base-Static 2-Displacement-Displacement1. Name; Type; Min; Max. Strain1; ESTRN: Equivalent Strain; 7.284e-08 Element: 4452; 1.059e-04 Element: 19056. Ammo Base-Static 2-Strain-Strain1. Name; Type. Displacement1{1}; Deformed shape. Ammo Base-Static 2-Displacement-Displacement1{1}. Name; Type. Design Insight1; Design Insight. Ammo Base-Static 2-Design Insight-Design Insight1. Name; Type; Min; Max. Factor of Safety1; Max von Mises Stress; 3.109e+00 Node: 20202; 1.251e+04 Node: 38403. Ammo Base-Static 2-Factor of Safety-Factor of Safety1.

# Conclusion